2DC Electric powered dust collectors per design specs.

Motor - 15 hp 3 ph 480V TEFC. With control panel soft start and stop/start buttons.

Motor - 19 amps full load.

Fan - Backward incline spark resisting.

Fan Performance - 2000 cfm @ 12” WC.

Filters - (4) 30” QX media 99.8% efficient @ .5 micron after initial dust cake.

Filters - MERV 12 initial rating.

Filters - 1,040 square foot of media, air to cloth ratio 1.92:1

Filters - Magnehelic Gauge for monitoring differential pressure across primary filters.

Filter Cleaning - Reverse jet pulse system with programmable timer.

Filters - HEPA final filter before fan.

Inlets - (2) 6” one each side of auger.

Waste Removal - Gravity discharge.

Frame - Telescopic drop legs for setting dump height.

Frame - Crane pick points

Frame - Fork lift pockets

Weight - 2400 lbs

Dimensions - 86”H x 40”W x 96”L in transport position.

Warranty - One year or 1500 hours whichever occurs first.