45DC Electric Skid Spec Sheet.

CYCLONE 45DC electric powered skid dust collector including the following equipment.

Motor - 100 HP 480V 3PH TEFC High Efficiency.

Motor - Control panel with soft start, Stop/Start, Main circuit disconnect, auger motor controls.

Fan Drive - Direct.

Filters - (60) 33” vertical top loading moisture resistant pleated cartridges.

Filter Efficiency - MERV 12 Initial Rating.

Filter Efficiency - 99.8% @ .5 Micron after initial dust cake.

Filtering Capacity - 45,000 cfm at 12” w.c.

Filtering Capacity - 17,160 sq. ft. of primary filter area for an air to cloth ratio of 2.62:1

Filter Cleaning - Solid state, programmable timer for reverse jet air pulse.

Baffle Sheets - Between each row of filters, protect next row from cross contamination.

Inlet Duct - (6) 20” ports, 2 each side, 2 rear.

Inlet Duct - Diverter sheets at each inlet to even out incoming dust laden air.

Frame - Solid and integral to dust collector.

Frame - Crane pick points.

Frame - Fork lift pockets.

Waste Discharge - Hydraulically driven unloading augers.

Dimensions - 128” High in transport, 101” Wide x 360” Long

High quality finish for long life. (Epoxy primer, Acrylic urethane top coat)

Warranty - One year or another 1500 hours whichever occurs first.