60DC FT4 Fifth Wheel Trailer Specs.

CYCLONE 60DC mobile diesel powered dust collector including the following equipment.

Engine - 190 HP Cummins FT4 Compliant.

Engine - Secured in enclosed lockable compartment.

Drive - Belt with Twin Disc PTO on engine.

Drive - Hydraulic engagement of PTO.

Filters - 75 Moisture resistant vertical top loading pleated cartridges.

Filtering Capacity - 60,000 cfm at 10” w.g.,

Fan - BI-445 spark resisting. .

Filter - 23,400 sq. ft. of QX media, air to cloth ratio 2.56:1.

Filter Efficiency - 99.8% @ .5 Micron after initial dust cake.

Filter Efficiency - MERV 12 initial rating.

Filter Cleaning - Reverse air pulse with adjustable timer.

Inlets - (8) 20” duct ports (3 each side, 2 rear).

Trailer - Fifth wheel with air brakes.

Fuel Capacity - 120 gallon.

Waste Discharge - Hydraulically driven unloading augers.

Finish - Epoxy primer, Acrylic urethane top coat.

Standard one year or 1500 hours whichever occurs first limited warranty.