6DC SPECIFICATIONS

Cyclone 6 DC diesel powered dust collector including the following equipment.

Engine – Yanmar FT4 Diesel.

Drive - Belt with Twin Disc PTO.

Filtering Capacity - 6,000 cfm @ 12” WC. Air to cloth ratio of 2.56:1

Filters - 9 vertical top loading cartridges.

Filters - 2340 sq. ft. QX primary media.

Filter Efficiency - 99.8% at .5 micron. after initial dust cake.

Filter Efficiency - MERV 12 Initial Rating.

Filter Cleaning - Reverse jet pulse system with programmable timer.

Inlets - (2) 16” duct ports (1 each side)

Trailer - (1) 5,000 lb capacity torsion axle with electric brakes.

Trailer - Hitch 2 5/16” ball coupler.

Trailer - DOT approved lighting and reflective recognition tape.

Engine Compartment - Enclosed and lockable.

Fuel Tank - 40 gallon capacity.

Waste Discharge - Hydraulic driven auger.

Dimensions - 116” H x 99” W x 178” L

Finish - Epoxy primer, Acrylic urethane top coat.

Dry weight - 4400 lbs

Warranty – One year or 1500 hours whichever occurs first.