9DC Specifications

Cyclone 9DC diesel powered mobile dust collector including the following equipment.

Engine - 24.4 HP Yanmar Diesel FT4 compliant.

Drive - Belt with Twin Disc PTO.

Filtering Capacity - 9,000 cfm @ 10” Wg.

Filters - 12 vertical top loading cartridges.

Filters - 3,120 sq. ft. QX primary media. Air to cloth ratio of 2.88:1

Filter Efficiency - 99.8% at .5 micron after initial dust cake.

Filter Efficiency - MERV 12 initial rating.

Filter Cleaning - Solid state programmable timer for reverse jet pulse system.

Inlets - (2) 20” duct ports (1 each side)

Trailer Axle - (2) 4000 lb Torsion with electric brakes.

Trailer Hitch - 2 5/16 coupler.

Trailer - DOT approved lighting and reflective recognition tape.

Engine Enclosure - Lockable compartment.

Fuel Tank - 40 gallon capacity.

Waste Discharge - Hydraulically driven unloading augers.

Dimensions -10’ H x 8’ 6” W x 18’ L

Finish - Epoxy primer, Acrylic urethane top coat.

Dry Weight - 6930 lbs

Warranty - One year or 1500 hours whichever occurs first limited.